# Technical Documentation

## Project Phase 5

RP2040 Bare-Metal RTOS — SysTick Integration and Preemptive Scheduling
Target: Raspberry Pi Pico (RP2040, Cortex-M0+) | Toolchain: ARM GCC + CMake + Ninja | Debugger: Cortex-Debug / OpenOCD

### 1. Objective Achieved

Phase 5 successfully transitioned the kernel from a cooperative scheduling model to a fully preemptive Real-Time Operating System.

By integrating the ARM Cortex-M0+ core SysTick timer, the OS now enforces a strict 1-millisecond execution time slice per task. Context switches are completely decoupled from application logic, handled asynchronously via hardware interrupts without requiring manual OS_Yield() calls from within the task loops.

### 2. Architectural Components Implemented

#### 2.1 SysTick Hardware Configuration

The core 24-bit SysTick down-counter was configured to generate a hardware interrupt at a fixed 1ms interval. Running at the RP2040's default 125 MHz clock (125,000,000 cycles/sec), the Reload Value Register (SYST_RVR) was calculated as (Frequency / 1000) - 1. The Control and Status Register (SYST_CSR) was updated to 0x07 to enable the counter, assert the exception request, and source the processor clock.

#### 2.2 Pico SDK-Compliant Exception Routing (isr_systick)

To ensure the ARM GCC linker correctly mapped the exception handler to Exception 15 in the vector table, the C handler was strictly named isr_systick to override the Pico SDK's weak default symbol.

Crucially, this handler does not perform the context switch. Its sole architectural responsibility is to assert the PENDSVSET bit (bit 28) in the Interrupt Control and State Register (ICSR at 0xE000ED04). This creates a safe handoff, queueing the lowest-priority PendSV exception to execute immediately upon exit from the SysTick handler.

#### 2.3 Task Isolation

Tasks (TaskA and TaskB) were refactored into pure, isolated execution loops. All cooperative OS_Yield() calls were removed. The tasks execute as standard while(1) loops, entirely ignorant of the underlying RTOS timeline and exception management.

### 3. Critical Boot Sequence Modifications

Transitioning to hardware interrupts exposed critical race conditions in the main() initialization sequence, which were resolved through strict operational ordering:

- TCB Pointer Pre-Initialization: The global tracking pointers current_tcb and next_tcb were explicitly mapped to &TCB_LIST[0] and &TCB_LIST[1] before interrupts were enabled. Failing to initialize these pointers prior to the first tick caused PendSV to attempt a register save at 0x00000000 (Boot ROM), resulting in a silent Hard Fault.

- Delayed Heartbeat Activation: The SysTick() configuration call was relocated to the absolute end of the main() boot sequence. The system timer is now only armed after the hardware stacks, control registers (PSP selection), and initial pointers are completely verified and loaded.

### 4. Debugging and Verification

Verification of the preemptive scheduler required abandoning instruction-level stepping, as GDB manipulation artificially delays the CPU while the hardware timer continues to fire, leading to debugger freezing and false state readings.

Verification was achieved via free-running execution (PAUSED ON USER REQUEST) with the following findings:

- Execution Volume Validation: In a free-running state, counters reached into the millions (e.g., 0x0f81bf27), proving stable, continuous task swapping.

- Asymmetric Loop Timing: TASKB_COUNTER incremented roughly 140 million times more than TASKA_COUNTER over the same duration. This accurately reflects the architectural reality that TaskA incurs peripheral bus access latency by writing to the GPIO SIO registers, whereas TaskB only executes rapid SRAM accesses. Both tasks received identical 1ms time slices, but TaskB physically completed more cycles per slice.

- Stack Frame Integrity: The debugger call stack explicitly reported <signal handler called>@0xfffffffd during task preemption. This confirms the Cortex-M0+ hardware unwound the fabricated Phase 4 stack frame correctly, executing an EXC_RETURN to Thread Mode using the Process Stack Pointer (PSP).

### 5. Next Step

With reliable time-sliced preemption active, the context-switching engine is feature-complete. The next phase will focus on Dynamic Task Management (N-Tasks) by replacing the hardcoded TCB_LIST toggle with a linked list or array-based scheduler capable of tracking an arbitrary number of ready tasks.