# Technical Documentation: Project Phase 1

## Current execution status

With the current code, the LED does not blink yet. The main() function calls Hardware_init() and then TCB_Initialization(). At this stage, TCB_Initialization() only calculates the top-of-stack memory address and stores it in TCB_LIST[0].topStackPtr. It does not transfer program execution to TaskA(). After that calculation completes, execution falls directly into the empty infinite loop at the end of main(). Because the CPU remains trapped in that loop, the LED never toggles. To make the LED blink immediately with the current project state, TaskA(); would need to be called before entering the final while(1) loop.

## 1. Objective achieved

Established a clean, modular bare-metal hardware foundation and the core kernel data structures required to support multi-threaded task management on the RP2040 (ARM Cortex-M0+), without relying on high-level abstraction frameworks.

## 2. Architectural components implemented

Low-Level Peripheral Control (Hardware_init): The project bypasses standard runtime libraries and interacts directly with hardware memory maps through custom pointer macros. The initialization sequence clears the reset state for the IO_BANK0 peripheral, waits for stabilization using a hardware status polling loop, configures the GPIO multiplexer so Pin 25 is mapped to the SIO block, and enables the output driver required to control the LED.

Decoupled Application Layer (TaskA): The execution logic has been separated into a standalone infinite task function, TaskA(), instead of being mixed into the hardware initialization path. This is an important structural step toward a scheduler-driven design.

Kernel Core Structure Memory Mapping (TCB_t): The Thread Control Block data structure is defined so that topStackPtr is the first field in the layout. This follows standard Cortex-M context-switch design practice, allowing the assembly context switcher to treat the TCB base address as the saved stack pointer location. A dedicated task stack array, such as TASKA_STACK, is allocated statically to provide isolated stack space for the task.

## 3. Pointer calculation architecture (TCB_Initialization)

The implementation correctly models a downward-growing stack, which matches the physical execution behavior of the ARM Cortex-M architecture. By taking the base address of the task stack array and adding STACK_SIZE, the code computes the highest address boundary of the reserved stack region. That address is then stored as the initial top-of-stack value for the task.

Memory Address Stack Space Map
-------------- -------------------
0x20040000 (Low) -> [ TASKA_STACK[0] ]
...
0x200401FC -> [ TASKA_STACK[127] ]
0x20040200 (High) -> [ topOfStack Base ] <-- Current calculation point

This calculation prepares the system for the next kernel milestone: constructing an artificial exception stack frame so that a later context restore can cause the CPU to begin executing TaskA() as if it were returning from an exception.

## 4. Next milestone

The next step is to extend TCB_Initialization() so it preloads a synthetic startup frame at the top of the task stack. On Cortex-M cores, exception return restores a fixed register frame from the stack, including xPSR, PC, LR, R12, and R0-R3. For a first task launch, the most important values are the initial xPSR and PC. The xPSR value must have the Thumb state bit set (bit 24 = 1), commonly initialized as 0x01000000, because Cortex-M0+ executes only Thumb instructions; clearing that bit causes a fault. The saved PC must point to TaskA() so that when the stack frame is restored, execution begins in the task entry function. The Cortex-M exception model uses automatic stack frame restoration on exception return, and the Thumb bit requirement is defined by the Arm architecture. [Arm Developer]() [Arm Developer]()

In practice, this means subtracting word offsets from the computed top-of-stack pointer and writing the startup register values into the correct positions of the fabricated stack frame. Once that frame is in place, a future scheduler or context-restore routine can load the saved stack pointer and trigger an exception return path that starts TaskA() safely. This is the key transition from storing task metadata to actually launching a thread context.