# Technical Documentation: Project Phase 2

## Objective Achieved

Successfully designed and implemented the Cooperative Kernel Inter-Process Communication (IPC) Trigger for the RP2040 using native ARMv6-M assembly language. This establishes the software-to-hardware interface required to invoke the operating system's scheduler on demand.

## Architectural Components Implemented

- Assembly-to-C Linkage: Utilized the .global OS_Yield directive to export the assembly execution symbol into the project's global namespace, enabling type-safe invocation from the C translation unit via an extern void OS_Yield(void); declaration.

- Asynchronous Exception Request Allocation: Targeted the core Interrupt Control and State Register (ICSR) located inside the private peripheral bus mapping of the Cortex-M0+ at address 0xE000ED04. Configured code to write a high bit to position 28 (PENDSVSET), which triggers an asynchronous request to the PendSV (Pended Service Call) exception vector.

- Memory and Pipeline Guarding: Integrated architectural barrier instructions (DSB and ISB) to enforce strict instruction sequencing. This guarantees that the memory store transaction to the NVIC completes entirely before the processor executes any subsequent pipeline stages or returns back to Thread Mode.

## The Next Step: The Core Context Switcher

Now that your C code can securely calculate stack pointer boundaries, pre-populate task tracking structures, and use OS_Yield to force the hardware into an exception state, you are ready for the most complex part of the entire project: Writing the actual PendSV_Handler context switcher in assembly.

This handler is what runs when OS_Yield triggers. It will physically swap the registers of TaskA with TaskB.